Fundamentals of Networking

Lab - 1

Exploring Cisco IOS

- Network Topology

# Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway |
| --- | --- | --- | --- | --- |
| Router1 | G0/0/0 | 192.168.1.1 | 255.255.255.0 | N/A |
| R1 | G0/0/1 | 192.168.2.1 | 255.255.255.0 | N/A |
| Switch1 | VLAN 1 | 192.168.1.2 | 255.255.255.0 | N/A |
| Switch2 | VLAN 1 | 192.168.2.2 | 255.255.255.0 | N/A |
| PC1 | NIC | 192.168.1.3 | 255.255.255.0 | N/A |
| PC2 | NIC | 192.168.2.3 | 255.255.255.0 | N/A |

Lab 1.1 Network Representation

- Identify common components of a network as represented in Packet Tracer.

Open a packet tracer file named 1.2.4.5 Packet Tracer - Network Representation.pka

  - Without entering into the internet cloud or intranet cloud, how many icons in the topology represent endpoint devices (only one connection leading to them)? 15

- Without counting the two clouds, how many icons in the topology represent intermediary devices (multiple connections leading to them) 11

- How many different types of media connections are used in this network topology? 4

- Compare and contrast LANs and WANs.

a. In the Packet Tracer network, how many WANs do you see? 2

b. How many LANs do you see? 3

Lab 1.2 Explore The Cisco IOS

- Set up the Network Topology

- Set up the network topology as per the above picture.

- Establish Basic Connections, Access the CLI, and Explore Help

  - Add a laptop, name it PC1 and connect it to Switch 1 using a console cable.

  - Click the Connections icon (the one that looks like a lightning bolt) in the lower left corner of the Packet Tracer window.

  - Select the light blue Console cable by clicking it. The mouse pointer will change to what appears to be a connector with a cable dangling from it.

  - Click PC1. A window displays an option for an RS-232 connection. Connect the cable to the RS-232 port.

  - Drag the other end of the console connection to the Switch 1 switch and click the switch to access the connection list.

  - Select the Console port to complete the connection.

- Establish a terminal session with Switch 1.

  - Click PC1 and then select the Desktop tab.

  - Click the Terminal application icon. Verify that the Port Configuration default settings are correct.

  - Click OK.

  - The screen that appears may have several messages displayed. Somewhere on the screen there should be a Press RETURN to get started! message. Press ENTER.

- Explore the IOS Help.

  - The IOS can provide help for commands depending on the level accessed. The prompt currently displayed is called User EXEC, and the device is waiting for a command. The most basic form of help is to type a question mark (?) at the prompt to display a list of commands.

Switch> ?

Q1- Which command begins with the letter ‘C’? Connect

  - At the prompt, type t and then a question mark (?).

Switch> t?

Q1- Which commands are displayed? telnet, terminal, traceroute

- At the prompt, type te and then a question mark (?).

Switch> te?

Q2- Which commands are displayed? telnet, terminal

This type of help is known as context-sensitive help. It provides more information as the commands are expanded.

- Explore EXEC Modes

- Enter privileged EXEC mode.

- At the prompt, type the question mark (?).

Switch> ?

Q3- What information is displayed for the enable command?

Turn on privileged commands

- Type en and press the Tab key.

Switch> en<Tab>

Q4- What displays after pressing the Tab key? enable

This is called command completion (or tab completion). When part of a command is typed, the Tab key can be used to complete the partial command. If the characters typed are enough to make the command unique, as in the case of the enable command, the remaining portion of the command is displayed.

Q5- What would happen if you typed te<Tab> at the prompt?The prompt went onto a new line with the “te”.

- Enter the enable command and press ENTER.

Q6- How does the prompt change? The prompt begins with the # sign

- When prompted, type the question mark (?).

Switch# ?

One command starts with the letter ‘C’ in user EXEC mode.

Q7- How many commands are displayed now that privileged EXEC mode is active? (Hint: you could type c? to list just the commands beginning with ‘C’.)

5 commands begin with “c”, and there are a total of 27 commands in the privileged exec mode.

- Enter Global Configuration mode

  - When in privileged EXEC mode, one of the commands starting with the letter ‘C’ is configure. Type either the full command or enough of the command to make it unique. Press the <Tab> key to issue the command and press ENTER.

Switch# configure

Q8- What is the message that is displayed? Configuring from terminal, memory, or network [terminal]?

- Press Enter to accept the default parameter that is enclosed in brackets [terminal].

Q9- How does the prompt change?Switch(config)#

This is called global configuration mode. This mode will be explored further in upcoming activities and labs. For now, return to privileged EXEC mode by typing end, exit, or Ctrl-Z.

Switch(config)# exit

Switch#

- Display the switch IOS image version.

enter user EXEC mode and use the show version command to display the IOS version for the switch.

Switch> show version

Q10- Which IOS image and version is currently in use by the switch?System image file is "flash:c2960-lanbasek9-mz.150-2.SE4.bin"

Version 15.0(2)SE4

- Add another laptop and connect the router and laptop using a mini-USB cable and explore the Exec modes.

Lab 1.3 Create a Basic Router Configuration

- Create a Basic Router Configuration

  - Enter to Global Configuration mode.

    - Connect to router via console session. Press Enter, then the user EXEC mode will appear.

    - Enter privileged EXEC mode by entering the enable command:

Router> enable

Router#

- Notice that the prompt changed to reflect privileged EXEC mode, then enter to Global Configuration mode by entering the configure terminal command:

Router# configure terminal

Router(config)#

Notice that the prompt changed to reflect Global Configuration mode

- Assign a name to a router.

  - Set a name for the router using hostname command:

Router(config)# hostname R1

R1(config)# exit

R1#

- Secure access to the console line.

To secure access to the console line, access config-line mode and set the console password to 123

R1# configure terminal

Enter configuration commands, one per line. End with CNTL/Z.

R1(config)# line console 0

R1(config-line)# password 123

R1(config-line)# login

R1(config-line)# exit

R1(config)# exit

%SYS-5-CONFIG_I: Configured from console by console

R1#

- Verify that console access is secured.

Exit privileged mode to verify that the console port password is in effect.

R1# exit

Router con0 is now available

Press RETURN to get started.

User Access Verification

Password:

R1>

- Secure privileged mode access.

Set the enable password to 456. This password protects access to privileged mode.

R1> enable

R1# configure terminal

R1(config)# enable password 456

R1(config)# exit

%SYS-5-CONFIG_I: Configured from console by console

R1#

- Verify that privileged mode access is secure

  - Enter the exit command again to log out of the router.

  - Press <Enter> and you will now be asked for a password:

User Access Verification

Password:

- The first password is the console password you configured for line con 0. Enter this password to go to user EXEC mode.

- Enter the command to access privileged mode.

- Enter the second password you configured to protect privileged EXEC mode.

- Verify your configuration by examining the contents of the running-configuration file:

R1# show running-config

- Notice that the console and enable passwords are both in plain text. This could pose a security risk if someone is looking over your shoulder or obtains access to config files stored in a backup location.

- Configure an encrypted password to secure access to privileged mode.

The enable password should be replaced with the newer encrypted secret password using the enable secret command. Set the enable secret password to 789

R1# config terminal

R1(config)# enable secret 789

R1(config)# exit

R1#

Note: The enable secret password overrides the enable password. If both are configured on the router, you must enter the enable secret password to enter privileged EXEC mode.

- Verify that the enable secret password is added to the configuration file.

Enter the show running-config command again to verify the new enable secret password is configured.

Note: You can abbreviate show running-config as

R1# show run

- Encrypt the enable and console passwords.

As you noticed in previously, the enable secret password was encrypted, but the enable and console passwords were still in plain text. We will now encrypt these plain text passwords using the service password-encryption command.

R1# config terminal

R1(config)# service password-encryption

R1(config)# exit

- Verify that the enable and console password is encrypted.

Enter the show running-config command again to verify the new enable and console password is encrypted.

R1# show running-config

- Configure a MOTD Banner

  - Configure a message of the day (MOTD) banner.

The network device IOS command set includes a feature that allows you to configure messages that anyone logging onto the router sees. These messages are called message of the day, or MOTD banners. Enclose the banner text in quotations or use a delimiter different from any character appearing in the MOTD string.

R1# config terminal

R1(config)# banner motd "This is a secure system. Authorized Access Only!"

R1(config)# exit

%SYS-5-CONFIG_I: Configured from console by console

R1#

- Save and Verify Configuration Files to NVRAM

  - Verify that the configuration is accurate using the show run command.

Save the configuration file. You have completed the basic configuration of the router. Now back up the running configuration file to NVRAM to ensure that the changes made are not lost if the system is rebooted or loses power.

R1# copy running-config startup-config

Destination filename [startup-config]?[Enter]

Building configuration...

[OK]

- Configure Switch1 and Switch2

You have completed the configuration on Router1. You will now configure Switch1 and 2. If you cannot remember the commands, refer to Parts 2 to 4 for assistance.

Configure Switch1 and 2 with the following parameters:

- Device name: S1 and S2 respectively

- Protect access to the console using a password of your preference.

pass1 and pass2

Configure an enable password and an enable secret password of your preference.

Enable password: passp1 and passp2

Enable secret: secret1 and secret2

- Configure an appropriate message to those logging into the switch.

- Encrypt all plain text passwords.

- Ensure that the configuration is correct.

- Save the configuration file to avoid loss if the switch is powered down.

- Configure Router and Switch Interfaces

  - Configure the GigabitEthernet 0/0/0 interface on R1.

    - Enter the following commands to address and activate the GigabitEthernet 0/0/0 interface on R1:

R1(config)# interface gigabitethernet 0/0/0

R1(config-if)# ip address 192.168.1.1 255.255.255.0

R1(config-if)# no shutdown

%LINK-5-CHANGED: Interface GigabitEthernet0/0/0, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface GigabitEthernet0/0/0, changed state to up

- It is a good practice to configure a description for each interface to help document the network. Configure an interface description that indicates the device to which it is connected.

R1(config-if)# description connection to R1

- Configure the remaining Gigabit Ethernet Interfaces on R1.

  - Use the information in the Addressing Table to finish the interface configurations for R1. For each interface, do the following:

    - Specify the interface to be configured.

    - Enter the IP address and subnet mask and activate the interface.

    - Configure an appropriate description.

    - Verify interface configurations.

- Configure the VLAN interface on S1.

  - Enter the following commands to address and activate the VLAN interface on S1:

S1(config)# interface VLAN 1

S1(config-if)# ip address 192.168.1.2 255.255.255.0

S1(config-if)# no shutdown

%LINK-5-CHANGED: Interface Vlan1, changed state to up

%LINEPROTO-5-UPDOWN: Line protocol on Interface Vlan1, changed state to up

- It is a good practice to configure a description for each interface to help document the network. Configure an interface description that indicates the device to which it is connected.

R1(config-if)# description VLAN on S1

- Configure the remaining VLAN Interfaces on S2.

  - Use the information in the Addressing Table to finish the VLAN interface configurations for S2.

    - Specify the VLAN interface to be configured.

    - Enter the IP address and subnet mask and activate the interface.

    - Configure an appropriate description.

  - Verify interface configurations.

- Verify the Configuration

  - Use verification commands to check your interface configurations.

    - Use the show ip interface brief command on R1, S1, and S2 to quickly verify that the interfaces are configured with the correct IP address and are active.

    - Use the show ip route command on R1 to view the current route.

    - Use the show running-config command on R1, S1, and S2 to check the configuration made.

- Configure PC’s

  - Click on PC1, then the Desktop tab, and select IP configuration. Enter the IP address and Subnet mask on the space provided based on the Addressing table.

  - Do the same thing for PC 2.

- Test connectivity

You should now be able to ping from PC1 to Router 1 G0/0/0 interface and from PC2 to Router 1 G0/0/1 interface.

From the command line on PC1, ping Router 1 G0/0/0 interface.

From the command line on PC2, ping Router 1 G0/0/1 interface.